SVST2_4_7I47

you need a 7I43 bitfile that has a name that ends in 7I47, other bitfiles will have the wrong pinout

you need to choose the appropriate bitfile in your .hal or .ini file

New Stepper driver wires:

Blue:   Common

Yellow: Step

Red:    Direction

Driver pinout

D25

1  - NC				1  - Black

2  - NC				2  - Black/White

3  - NC				3  - Brown

4  - NC				4  - Brown/White

5  - input5				5  - Red

6  - input6				6  - Red/White

7  - input7 				7  - Red/Black

8  - input8				8  - Orange

9  - x direction		P2-4	9  - Orange/White	P2-4 high during X+ move

10 - A Step		    	P2-19	10 - Orange/Black

11 - Z Step		    	P2-13	11 - Pink		P2-13

12 - Y Step		    	P2-7	12 - Pink/Black		P2-7

13 - X Step		    	P2-1	13 - Yellow		P2-1

14 - ENABLE ALL		P2-6	14 - Yellow/Black	stays low (GND)

15 - A Park				15 - Green

16 - Z Park				16 - Green/White	goes low during z step pulse streams

17 - Y Park				17 - Green/Black	goes low during y step pulse streams

18 - X Park				18 - Lt Green		goes low during x step pulse streams

19 - A Direction		P2-22	19 - Blue

20 - Z Direction		P2-16	20 - Blue/White		P2-16 low during Z+ move

21 - Y Direction		P2-10	21 - Purple		P2-10 low during Y+ move

22 - INternal VCC +5v		22 - Purple/White	available +5v for external use

23 - OPT VCC (Input)		23 - Grey		possibly tie to +5v but not yet

24 - Internal GND		P2-3	24 - Grey/Black		tie to GND

25 - OPT GND (Input)	P2-9	25 - White		tie to GND

PENDANT WIRING      (P3-P4 on 7i47)

DB25  Color on      7I47	Color in    Function

PIN   Pendant       PIN	Control

1  - Black	 	NC

2  - Black/White 	NC

3  - Brown	 	P4-15	Black		PAUSE/RESUME

4  - Brown/White 	P3-12	Brown		SPINDLE

5  - Red	 	P3-15	Red		ESTOP

6  - Red/White   	P4-12	Orange		res .01

7  - Red/Black	P4-9	Yellow		res .001

res .0001	NC (default)

8  - Orange		P3-9	Green		A Axis

9  - Orange/White	P3-7	Blue		Z Axis

10 - Orange/Black	P3-4	Purple		Y Axis

11 - Pink		P3-1	White		X Axis

12 - Pink/Black	P4-4	Black		MPG B

13 - Yellow		P4-1	Brown		MPG A

14 - Yellow/Black	P4-6	Red |		+5v mpg

15 - Green		P4-14	    |		+5v xyz switch

16 - Green/White	P4-17	    |		+5V res switch

17 - Green/Black		Orange

18 - Lt Green			Yellow

19 – Blue			Green

20 - Blue/White		Blue

21 - Purple			Purple

22 - Purple/White		NC

23 - Grey			NC

24 - Grey/Black		NC

25 - White		P4-3	White		GND mpg

LIMIT SWITCHES & SPINDLE MOTOR (P3 on 7I43)

Pin   Funct    Color    Signal

50 ----------  Red      GND

49 ----------  Purple   +5 SPINDLE

47 -- IO47 --  Brown    X+

45 -- IO46 --  Black    X-

43 -- IO45 --  WHite    Y+

41 -- IO44 --  Blue     Y-

39 -- IO43 --  Green    Z+

37 -- IO42 --  NC       Z-

35 -- IO41 --  NC       A+

33 -- IO40 --  NC       A-

31 -- IO39 --  Yellow   SPINDLE ON

29 -- IO38 --  ( Touch Probe connection )

27 -- IO37 --  Yellow    SCALE        was IO10 (P4-09) Yellow Pin 21 on 7i43

25 -- IO36 --  Orange    SCALE        was IO12 (P4-12) Orange Pin 25 on 7i43

23 -- IO35 --  Black     Pause/Resume was IO14 (P4-15) Black  Pin 29 on 7i43

21 -- IO34 --  Green     Program Run

19 -- IO33 --  Brown

17 -- IO32 --  Red

15 -- IO31 --  Blue

13 -- IO30 --  Purple

11 -- IO29 --  White

09 -- IO28 --

07 -- IO27 --

05 -- IO26 --

03 -- IO25 --

01 -- IO24 --