| # Technical comparison |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type | MMC | RS-MMC | MMC Plus | SecureMMC | SD | SDIO | miniSD | microSD |
| SD Socket | Yes | Mechanical adapter | Yes | Yes | Yes | Yes | Electro-mechanical adapter | Electro-mechanical adapter |
| Pins | 7 | 7 | 13 | 7 | 9 | 9 | 11 | 8 |
| Form factor | Thin | Thin/short | Thin | Thin | Thick (exceptions possible) | Thick | Narrow/short/thin | Narrow/short/extrathin |
| Width | 24 mm | 24 mm | 24 mm | 24 mm | 24 mm | 24 mm | 20 mm | 11 mm |
| Length | 32 mm | 18 mm | 32 mm | 32 mm | 32 mm | 32 mm+ | 21.5 mm | 15 mm |
| Thickness | 1.4 mm | 1.4 mm | 1.4 mm | 1.4 mm | 2.1 mm (exceptions possible) | 2.1 mm | 1.4 mm | 1 mm |
| SPI mode | Optional | Optional | Optional | Required | Required | Required | Required | Optional |
| 1 bit mode | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| 4 bit mode | No | No | Yes | ? | Optional | Optional | Optional | Optional |
| 8 bit mode | No | No | Yes | ? | No | No | No | No |
| Xfer clock | 0–20 MHz | 0–20 MHz | 0–52 MHz | 0–20 MHz? | 0–25 MHz - 0–50 MHz | 0–25 MHz | 0–25 MHz? | 0–25 MHz? |
| Max XFER | 20 Mbit/s | 20 Mbit/s | 416 Mbit/s | 20 Mbit/s? | 100 Mbit/s - 200 Mbit/s | 100 Mbit/s | 100 Mbit/s | 100 Mbit/s |
| Max SPI XFR | 20 Mbit/s | 20 Mbit/s | 52 Mbit/s | 20 Mbit/s | 25 Mbit/s | 25 Mbit/s | 25 Mbit/s | 25 Mbit/s |
| DRM | No | No | No | Yes | Yes | N/A | Yes | Yes |
| User encrypt | No | No | No | Yes | No | No | No | No |
| Simplified Spec | Yes | Yes | No | Not yet? | Yes | Yes | No | No |
| Memb cost | $2500/yr (not required) |  |  |  | $1500/yr (appears required) |  |  |  |
| Spec cost | $500 |  | ? | ? | Member | Member | Member | Member |
| Host license | No | No | No | No | $1000/yr+memb |  |  |  |
| Mem card royalties | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| I/O card royalties | N/A | N/A | N/A | N/A | N/A | $1000/yr+memb | N/A | N/A |
| Open source compatible | Yes | Yes | Yes? | Yes? | SPI only | SPI only | SPI only | SPI only |
| Type | MMC | RS-MMC | MMC Plus | SecureMMC | SD | SDIO | miniSD | microSD |

Table data compiled mostly from simplified versions of MMC and SDIO specifications and other data on SD card and MMC association web sites. Data for other card variations is interpolated.